Here is the tutorial of the handsim project:

- Environment Setup

  - Software Installation

- Set up the Vuzix hmd:

1. In the attached folder, double click the “VR_Manager_3.1.zip”, uncompressed this and install the VR_Manager_Installer.exe.

2. Double click the “Vuzix_SDK_3.1.1.zip”, then read the read me file. Install the “Vuzix_Eyewear_SDK-3.1.1.exe” first, then install the “AR_Ext_Setup_1.0.exe”.

3.The SDK installer creates the following two directories:

C:\IWEARSDK - SDK installation directory

C:\IWEARSDK\docs - SDK documentation directory

4. Build Instructions:

Debug builds are designed to run in windowed mode to ease debugging, while release builds run in full screen.

Add the following folder paths to your include directories:

<Vuzix SDK Path>\inc	------>	<Platform SDK Path>\include

<Vuzix SDK Path>\lib	------>	 <Platform SDK Path>\lib

- Set up the cybergloves:

- Install VirtualHand SDK from the attached files: “VirtualHandSDKSetupV2_10_0.exe”

- Install VirtualHand for Motion Builder in the attached files:

” VH_for_MB_7_5_ext2_setup.exe”

- From the Start Menu select Authorize.

- Copy the Site Code and send an email to support@cyberglovesystems.com with it asking that you would like a Site Key. (this should take one or two days)

- Authorize the software when you get the Site Key.

- Use the programmer's guide to get set up.

Note: All the SDK file has been included into an independent folder, so except installing the software, nothing more should be worried about.

  - Hardware Setup

There are three different devices we need to set up before running the code.

Cyberglove

HMD

Mocap System

All the devices we need:

USB extension cabal

Arm markers sets

HMD glassed with markers

VGA extension cabal

Cyberglove

    - HMD

First: Put the HMD on the chair and in center of the mocap area.

Second: connect the usb extension line and the VGA extension cable to the HMD.

Third: Connecting the cable and line to the desktop.

Connect to the right slot

    - Cyberglove

First: Put the cuff on another chair.

Second: Open the cyberglove box.

Cyberglove Sensor cable

Nylon glove inserts and Tracker accessory kit

Cyberglove data glove

Interface module

Serial conversion box with power converter

AC/DC power supply

Then put the glove on the chair, too

Third: connect the power convert to the desktop

Fourth: connect the power supply

Fifth: connect the power supply to the plug-base.

Finally: turn on the cybergleve

    - Mocap

Find the desktop which controls the mocap system and follow the instruction of the attached file.

- Code implementation

2.1 CCyberglove

CCyberglove is the class used to connect the cybergolve hardware, to get all the 23 dof’s rotation data and used into our own hand model.

2.1.1 Parameters

| vhtIOConn *gloveDict; | Virtual hand connection class |
| --- | --- |
| vhtCyberGlove *glove; | Virtual hand glove data glass |
| std::vector<double> g_d_rotate_array; | 23 rotation array |

2.1.2 Methods

| int connect(); | Connect to the cyberglove hardware |
| --- | --- |
| void updateRotation(); | Refresh the 23 dof rotation every frame |
| std::vector<double> getRotationArray(); | Get the rotation array from the 23 dof |

2.2 CHMD

2.2.1 Parameters

| float m_EyeSeparation; | Intraocular Distance: aka, Distance between left and right cameras. |
| --- | --- |
| float m_FocalLength; | Screen projection plane: aka, focal length(distance to front of virtual screen). |
| GLint m_screenWidth, m_screenHeight; | Screen width and height, same with the PC screen |
| HANDLE m_StereoHandle; | Handle to connect to the HMD stereo mode |
| bool m_StereoEnabled; | Flag to switch the Stereo mode |

2.2.2 Methods

| int connect(); | Function to connect the HMD |
| --- | --- |
| void SetViewingFrustum( int Eye , GLfloat fovy, GLfloat nearZ, GLfloat farZ, GLfloat aspect); | Set the frustum based on view mode and other parameters to set up the view |
| void prerenderSetting(int Eye); | Set which eye to render |
| void SetStereoViewport(int Eye); | Set view port based on the mode |
| void toggleStereoMono(); | Change between the Mono and Stereo Mode |
| void increaseEyeSeparation(); void decreaseEyeSeparation(); void increaseFocalLength(); void decreaseFocalLengthn(); | Change the Separation and the Focal length to set the 3D effect |

2.3 CMocap

2.3.1 Parameters

| char *hostname ; | The hostname to connect the server PC |
| --- | --- |
| int unlabMarkers; | Numbers of the markers in the VR world |
| ViconDataStreamSDK::CPP::Client rtmcClient; | real-time mocap client |
| _array<rtmcSubjectForDirectControl> m_rtmc_Subjects; | real-time mocap subjects |
| double cameraMatrix[16]; | Position and rotation Matrix of the camera |
| Void* m_rtmc_hThread; | thread handle for real-time mocap |
| bool m_b_rtmc_active; | mocap server connected |
| _array<CMeanFilterSE3> m_rtmc_filterT; | filter for each subject |

2.3.2 Methods

| bool rtmcConnect() | Function to connect the server |
| --- | --- |
| void rtmcDisconnect() | Function to disconnect the server |
| void rtmcUpdateData() | Update the position and rotation for each object |
| double* getCameraMatrix() | Return the camera matrix |
| SE3 getHandBasePose() | Return the hand model matrix |

- Start Manager Software before run the project

There are two manager software you should start before running the code. One is the cyberglove manager and the other is the HMD manager. Also you need to change the resolution of the screen.

For the desktop in the lab, they are just in the start menu

Cyberglove Manager

HMD Manager

If they are not in the start menu, you can also find them in the program menu

Besides, change the resolution of the screen. Once set up the HMD.

Right click the mouse and click the NVIDIA control panel.

Then choose the “The screen on both displays” and “WR902 + Dell 2007FP”, and click Apply.

- Attached documents

There are several attached documents in the folder.

| VR_Manager_3.1.zip | HMD manager |
| --- | --- |
| Vuzix_SDK_3.1.1.zip | HMD SDK |
| VirtualHandSDKSetupV2_10_0.exe | Cyberglove install software |
| VH_for_MB_7_5_ext2_setup.exe | Cyberglove motion builder |
| load_BMP_function.cpp | Function about how to load bmp texture |
| Junggon.cal | Junggon’s hand calibration data |
| Creating Your Own Objects.docx | Tutorial about how to set up the mocap |
| calibration guide-v4-4.pdf | File about how to calibrate people’s hand |